Serial Communication

Code:

#include<p18F4550.h>

#include<stdio.h>

#define Fosc 48000000UL

void InitUART(unsigned int baudrate)

{

TRISCbits.RC6 = 0; //TX pin set as output

TRISCbits.RC7 = 1; //RX pin set as input

SPBRG = (unsigned char)(((Fosc /64)/baudrate)-1);

BAUDCON = 0b00000000; //Non-inverted data; 8-bit baudrate generator

TXSTA = 0b00100000; //Asynchronous 8-bit; Transmit enabled; Low speed baudrate select

RCSTA = 0b10010000; //Serial port enabled; 8-bit data; single receive enabled

}

void SendChar(unsigned char data)

{

while(TXSTAbits.TRMT == 0); //Wait while transmit register is empty

TXREG = data; //Transmit data

}

void putch(char data)

{

SendChar(data);

}

unsigned char GetChar(void)

{

while(!PIR1bits.RCIF); //Wait till receive buffer becomes full

return RCREG; //Returned received data

}

void main(void)

{

InitUART(9600);

printf("\r\nHello MicroPIC-18F: Enter any Key from Keyboard\r\n");

while(1)

{

printf("%c ",GetChar()); //Receive character from PC and echo back

}

while(1);

}

LCD using PIC

#include<p18f4550.h>

#define EN LATAbits.LA1

#define RS LATAbits.LA0

#define Dataport LATB

void msdelay(void);

void LCD_CMD(unsigned char value);

void LCD_DATA(unsigned char value);

void main(void)

{

TRISB =0x00;

TRISAbits.RA0 = 0; //RS pin

TRISAbits.RA1 = 0; // EN pin

LCD_CMD(0x38);

msdelay();

LCD_CMD(0x06);

msdelay();

LCD_CMD(0x0C);

msdelay();

LCD_CMD(0x01);

msdelay();

LCD_CMD(0x80);

msdelay();

LCD_DATA('N');

msdelay();

LCD_DATA('M');

msdelay();

LCD_DATA('I');

msdelay();

LCD_DATA('E');

msdelay();

LCD_DATA('T');

msdelay();

while(1);

}

void msdelay(void)

{

int i,j;

for(i=0;i<100;i++)

for(j=0;j<1000;j++);

}

void LCD_CMD(unsigned char value)

{

Dataport=value;

RS=0;

EN=1;

msdelay();

EN=0;

}

void LCD_DATA(unsigned char value)

{

Dataport=value;

RS=1;

EN=1;

msdelay();

EN=0;

}

PWM 25%,50% For DC Motor

//Pulse width Modulation for 25% Duty cycle

#include<P18F4550.h>

#define datapin PORTCbits.RC2

void main (void)

{

T2CON= 0x10;

PR2 = 62;

CCPRIL =  15;

CCPICON =  0x2C;

TRISCbits.TRISC2 = 0;

TMR2 = 0x00;

T2CONbits. TMR2ON = 1;

while (PIRlbits.TMR2IF == 0);

T2CONbits.TMR20N =  0;

PIRlbits.TMR2IF =  0;

}

//Pulse width Modulation for 50% Duty cycle #include<P18F4550.h> #define datapin PORTCbits.RC2 void main (void) { T2CON= 0x10; PR2 = 62; CCPRIL = 30; CCPICON = 0x3C; TRISCbits.TRISC2 = 0; TMR2 = 0x00; T2CONbits. TMR2ON = 1; while (PIRlbits.TMR2IF == 0); T2CONbits.TMR20N = 0; PIRlbits.TMR2IF = 0; }

Simple Square Wave Program

#include<p18f4550.h>

void msdelay ();

void main(void)

{

TRISDbits.TRISD3=0;

TICON=0x80;

while (1) {

PORTDbits.RD3=1;

msdelay ();

PORTDbits.RD3=0:

msdelay ();

}

}

void msdelay (void)

{

TICON=0x80;

TMR1=0xFED4;

T1CONbits.TMR1ON=1;

while (PIR1bits.TM1IF==0);

PIR1bits. TMR1IF=0;

TICONbits.TMR1ON = 0;

}

LED Relay Buzzer Using Pic

#include<p18f4550.h>

void msdelay (void)

.{

T0CON = 0x00;

TMR0H=0XFA;

TMR0L = 0x93;

T0CONbits.THR0ON=1;

while (INTCONbits.T0IF==1

INTCONbits.T0IF=0;

T0CONbits.TMR0ON=0;

}

void main(void)

{

TRISB = 0x00;

while (1)

{

PORTB = 0x00;

msdelay();

PORTB = 0xFF ;

msdelay();

}  }

Experiment 1 (8051)

Practical 1 (A) (MEMORY ORGANISATION) ;

org 0000h

mov r0,#50h;

mov r1,#04h;

mov A,#@r0;

INC r0;

ADC_LOOP:ADD A,@r0;

INC r0;

DJNZ r1,ADC_LOOP;

mov 60h,A;

SJMP $;

END

Practical 1 (B) (DATA TRANSFER) ;

org 0000h

mov r0,#50h;

mov r1,#60h;

mov r2,#04h;

COPY_LOOP:

mov A,@r0;

mov @r1,A;

INC r0;

INC r1;

DEC r2;

JNZ COPY_LOOP;

SJMP $

END

Experiment 2A : Seven Segment Display using 8051/P89V51RD2

Practical 2A (7-seg Display using 8051/P89V51RD2)

#include<reg51.h>

void delay()

{

int i;

for(i=0;i<30000;i++);

}

void main()

{

unsigned char seg[10]={0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,0x07,0x7f,0x6f};

int i;

while(1)

{

for(i=0;i<10;i++)

{

P2=seg[i];

delay();

}}}

Experiment No. 2 (B)

Practical 2B (To study port structure of 8051 Microcontroller,

Parallel Port Interfacing of LEDs - Flashing, Counter, BCD, HEX, Display of chara.)

#include <reg51.h>

void delay_5ms(void);

void main(void)

{

while(1)

{

P1 = 0x00;    // LED OFF

delay_5ms();

P1 = 0xFF;    // LED ON

delay_5ms();

}

}

void delay_5ms(void)

{

TMOD = 0x01;  // Timer0 Mode1

TH0  = 0xEE;

TL0  = 0x00;

TR0 = 1;      // Start timer

while(TF0 == 0);  // Wait for overflow

TR0 = 0;

TF0 = 0;      // Clear flag}

Experiment No. 3A

Practical 3A (Square Wave)

#include <reg51.h>

void tdelay(void);

void main(void)

{

while(1)

{

P1 = 0x00;

tdelay();

P1 = 0xFF;

tdelay();

}

}

void tdelay(void)

{

TMOD = 0x10;

TL1=0x1A;

TH1 = 0xFF;

TR1 = 1;

while(TF1 == 0);

TR1 = 0;

TF1 = 0;

}

Practical 3A (Triangular Wave)

#include <reg51.h>

void main(void)

{

P1 = 0x00;

while(1)

{

while (P1!=0xFF)

P1++;

while (P1!=0x00)

P1--;

}

}

Experiment No. 3B (Stepper Motor)

Practical 3B (Stepper) Full Step Clkwise

#include <reg51.h>

void msdelay(void);

void main(void)

{

while(1)

{

// FULL STEP SEQUENCE

P2 = 0x09;   // 1001

msdelay();

P2 = 0x0C;   // 1100

msdelay();

P2 = 0x06;   // 0110

msdelay();

P2 = 0x03;   // 0011

msdelay();

}}

void msdelay(void)

{

unsigned int i, j;

for(i = 0; i <100; i++)

for(j = 0; j < 1275; j++);   // Approx 1 ms delay

}

//Anti-CLK wise

#include <reg51.h>

void msdelay(void);

void main(void)

{

while(1)

{

// FULL STEP SEQUENCE

P2 = 0x03;   // 1001

msdelay();

P2 = 0x06;   // 1100

msdelay();

P2 = 0x0C;   // 0110

msdelay();

P2 = 0x09;   // 0011

msdelay();

}}

void msdelay(void)

{

unsigned int i, j;

for(i = 0; i <100; i++)

for(j = 0; j < 1275; j++);   // Approx 1 ms delay

}